ROV flight protocols – Pier 59

This document provides instructions for the following main steps required to safely conduct ROV flight operations at Pier 59

Assessing topside and underwater conditions

- Check weather, tides, and sea surface state before initiating flight protocol. There are a variety of ways to obtain information about the tide, wind. Two options include

  - Tides app on iPhone,

  - WindFinder app on iPhone

  - Do not operate on a tide lower than 1 foot (the ramp will be too steep / can’t reach the sea surface)

  - Do not operate with winds greater than 15 kts if out of the north.

  - Winds out of the south are largely blocked by Pier 59, though you may have water movement / chop.

- Check water conditions via docking station camera for visibility at-depth

  - http://10.59.165.221/extensionv2/cockpitlite/#/

  - Do not fly if you have less than 1m of visibility

Set up topside flight area

- Set up a pop-up tent if required to alleviate rain / bright sun or glare off the water.

- Unlock ramp power box lever using AB1 key and power the ramp via moving the switch up to the “on” position.

- Test both directions of the ramp to verify functioning -- descend the ramp slightly, ascend the ramp slightly.

- Move cart to the dive ramp, positioned such that the cart is to the right of the end of the ramp.

- Plug in the cart to the 120V power outlet.

- Plug in the HDMI of the TV to the ROV computer.

- Plug in the ROV computer to the power strip within the cart.

- Turn on and sign into the computer, verify TV duplicating computer screen.

Power ROV

- Check the status of the ROV battery with the voltage reader.

- Remove the vent plug and set somewhere safe.

- Use the small Blue Robotics enclosure key to gently open the ROV battery enclosure. Pull the rear end cap off.

- Check the status of silicone lubrication on the two rear endcap o-rings

  - If they are dry and/or if removal of the end cap is challenging, you can either:

    - Remove the o-rings with the o-ring pick, clean them with a dust cloth, apply silicone grease, reapply the o-rings, or

    - Apply a small amount of silicone grease to the outside of the x2 o-rings

- Connect the ROV: attach the ROV battery’s cell balancer connector (white) first, then connect the main connector (yellow). You should hear an initial chime when the battery is first connected, and a second chime approximately one minute later when the Pi and Navigator have fully booted.

- Gently replace the end cap; the cap should rest flush against the tube, with two solid black lines forming inside where the o-rings make contact with the tube.

  - Care is required when re-connecting the end cap – ensure that all cables are out of the way and that the end cap can seat properly, then match the rectangular notch (located about a cm up from the bottom) to fit the groove in the acrylic tube.

- Attach and tighten the vent plug.

Stage for launch at the dive ramp

- Double-check that the tether is securely clipped to ROV (i.e., that the tether is both plugged in, and that its strain relief clip is attached – should you have to manually pull in the ROV mid-flight, this clip will better protect the tether from damage.

- You require x2 people familiar with ramp operations – a Pilot and a Copilot – both personnel are required to wear a Personal Floatation Device (PFD) -- found on the inside door of the closet adjacent to the dive ramp – when out on the dive ramp.

- Pilot carries the ROV, Copilot carries the tether – they both walk out to the end of the ramp and place both items at the end of the ramp.

- The Copilot stays at the end of the ramp; the Pilot returns to the cart, closing but not locking the ramp (keep the unlocked lock threaded in its spot)

- The Copilot hands the tether up and over to the Pilot – the Pilot will verify they have the tether via a verbal “Got it!” before the Copilot releases the tether.

- The Pilot sets the tether on the ground to the left of the cart.

Connect to the ROV

- Connect the yellow cable to the FXTI box (align the bump along the interior rim of the plug with the white line on the spool connector).

- Connect the short yellow cable to the tether spool

- Ensure the yellow cable is routed such that does not impede the tether, i.e., ensure the tether can freely unspool.

- Connect the USB-A cable from the FXTI box to the flight laptop’s USB-A port.

- Connect the Xbox controller to flight laptop’s USB-A port.

- Access BlueOS running the ROV’s Raspberry Pi by opening a Google Chrome tab and proceeding to 192.168.2.

  - Verify BlueOS connection, check ROV voltage

- Open ROV laptop desktop program Cockpit

  - Verify camera feed

  - Arm vehicle

  - Verify control: check gain, adjust to 30%

  - Briefly check thrusters

  - Cycle light up and down

- The vehicle is now ready to deploy.

Deploy and fly the ROV

- The Copilot is staged on the ramp with a

- The Pilot lowers ramp with Copilot and ROV just above water level.

- Copilot makes a final check to ensure vent plugs are in place and tight.

- Copilot issues a verbal “Are you ready?” to the Pilot, Pilot verifies they the vehicle is armed and ready.

- Copilot gently sets the ROV in the water.

- Pilot motors the ROV on the surface northwest about 10 meters away from the ramp and ascent piling; visually confirm ROV flight controls are responding as expected.

- Descend the ROV

  - Our CCR team surveys with the ROV at 20-40% thruster gain, depending upon current.

  - 20-40% gain works well for smaller movements when investigating and exploring.

  - 40-70% gain can be utilized to cover longer distances quickly.

  - You will likely not need to run the thrusters at 100% -- this is where you will have the least control and ability to react quickly, especially given the less than crystal clear visibility.

  - Operate the x2 downward and x2 forward lights as desired.

Tether management

- The Copilot manages tether – let out slack as needed, pull in slack. Communicate so the Copilot understands where the ROV is, and where the Pilot is wanting to go, such that the Copilot can anticipate the need to spool out or recover tether slack. Communicate and anticipate for longer runs such as flying to the kelp in-between Piers 60 & 62.

  - The ROV’s tether is buoyant – it your visual guide – where the tether drops down towards the seafloor indicates where the ROV is. You have no other form of ROV localization. However, the placement of the tether at the surface can be shifted by wind, current, and other water movement.

  - Be especially aware and alert when the tether is bunched against the pilings – there are hazards that can snag the tether, especially further under the pier.

  - Do not let excess amount of tether gather at the surface – actively manage the tether.

  - If the ROV’s forward movement is sluggish, or it can’t move forward . . . you might need to let out tether.

  - If you get the tether wrapped around something, you can likely undo that wrap. Stay calm, be patient, and conduct your movements carefully.

  - When navigating back to the ramp in dark water, if you don’t know which was to go: turn around, pivot the camera up, and find the tether – that is your path home – follow it.

    - Sun glare shining down into the water can obscure the yellow tether.

  - You can always ascend your vehicle to the surface to visually locate it.

Docking, undocking, and wirelessly charging the ROV

- Align the ROV in front of the docking station.

- Place the target cross in the center of Cockpit.

- Gently motor forward such that the stab of the UWCS enters the receiver.

- A higher gain of 70% or greater is recommended to ensure the ROV is fully docked when thrust forward.

- Release the ROV via switching to Manual flight mode; observe the ROV affixed in place, i.e., it should jostle extensively but display a clear hold to the station (i.e., doesn’t float up/away).

Retrieve ROV

- Pilot has the ROV on the surface and is ready to land.

- Copilot verifies ramp power, grabs PFD, lowers ramp to just above water level, unlocks gate, and heads out to the end of the ramp.

- Pilot approaches ramp, switches to Manual, makes final approach, side turn for easy removal.

- Copilot, with one hand on rail, puts the other hand on the ROV and issues a verbal “Got it!” to the Pilot – the Pilot then responds “Manual!” to verify the ROV is in ready to retrieve (pulling the ROV out of the water in Stabilize or Depth Hold will result in the thrusters continuing to fire in an attempt to minimize ROV movement, i.e., you will get wet as the ROV is pulled out of the water).

- Copilot lifts ROV out of the water.

- Pilot Disarms vehicle and raises ramp.

- Pilot spools up and clears tether.

Post-flight care

- Take the ROV and tether to the back of Pier 59 and spray thoroughly with freshwater from the hose.

  - Focus particularly on thrusters, areas with wire connections, and any exposed metal.

- Once ROV has been rinsed, remove vent plug from battery enclosure and disconnect battery

- Reattach battery enclosure panel.

- Replace vent plug.

- Stow ROV and tether in the cart and wheel to old PCR habitats in shark holding.

Miscellaneous

- Don’t leave the ROV electronics enclosure exposed to direct sunlight for too long as UV can turn the acrylic opaque—place a towel over top on sunny days during standby (give the ROV a hat!).

- Whenever you need a better angle to perform work on the ROV (e.g., when removing/attaching the battery), avoid supporting the full weight of the ROV on its “nose,” any wires, or other small points of contact.

- Avoid hard landings both on land and in the water to avoid degrading the payload skid (don’t drop the ROV!)

Troubleshooting

- Most software issues can be resolved by cycling the power (turning the ROV off and on)

  - In BlueOS, click the power icon at the bottom of the left panel

  - Select “Restart Autopilot” from the pop-up and allow the ROV a minute or two to correct

  - Should that fail to resolve the issue, select “Reboot” from the pop-up and wait for the ROV to power cycle, listening for the chimes.

  - You can also shutdown the ROV via BlueOS, unplug the battery, wait a minute, and re-plug the battery. This is a full restart.

- Camera stream issues

- Unable to open BlueOS

  - Close the browser with BlueOS

  - Unplug FXTI USB from the laptop and plug back in

  - Reopen BlueOS

- Controller not connecting to Cockpit

  - Unplug and replug into laptop